PAL / NTSC Video IP Core

User & Reference Guide  with Simulation Reports

Repository: robokks/video-ip-core-  |  GHDL --std=08  |  All 14 checks PASS

This document covers all Video IP core variants in the tv/ folder. Sections include file load order, generics/ports reference, pattern descriptions with visual DAC output strips, actual simulation console logs, and waveform captures from GHDL VCD output.

# 1.  Simulation Results Summary

| Testbench | Tests / Checks | Result | Sim Time |
| --- | --- | --- | --- |
| tb_pal_tv_crt50_top | 5 pattern checks | ALL PASS | 20.48 ms |
| tb_pal_tv_interlaced_top | 4 checks + sync shape | ALL PASS | 46.40 ms |
| tb_pal_tv_prog25_top | 5 pattern checks | ALL PASS | 12.80 ms |
| tb_pal_bram_loader | 3 scenarios (24 writes verified) | ALL PASS | 1.035 µs |
| tb_pal_tv_bram_top | sel=0 bars + sel=4 BRAM F1+F2 | ALL PASS | 46.40 ms |
| tb_pal_sel5 | 4 ball-position checks | ALL PASS | 80.00 ms |
| tb_new_patterns | 5 checks (sel 6–9, ball ports) | ALL PASS | 3.37 ms |
| tb_video_bram_top | PAL+NTSC sel=5 + level sweep | ALL PASS | 80.00 ms |

Total: 14 testbench checks across 8 testbenches.  Zero failures.

# 2.  Repository & Compile Order

tv/

├── pal_timing.vhd               [1] Shared H/V counter — ALWAYS compile first

├── pal_sync_gen.vhd             [2] Shared sync decode (Opt 1 & 3 only)

│

├── opt1_crt50/

│   ├── pal_tv_crt50_top.vhd    [3a] Option 1 top

│   └── tb_pal_tv_crt50_top.vhd     sim only

│

├── opt2_interlaced/

│   ├── pal_csync_il.vhd        [3b] Interlaced composite sync — compile before top

│   ├── pal_tv_interlaced_top.vhd [4b] Option 2 top

│   └── tb_pal_tv_interlaced_top.vhd

│

├── opt3_prog25/

│   ├── pal_tv_prog25_top.vhd   [3c] Option 3 top (needs pal_sync_gen)

│   └── tb_pal_tv_prog25_top.vhd

│

└── bram/

├── pal_bram_loader.vhd     [5]  Host FIFO-to-BRAM loader (standalone)

├── pal_tv_bram_top.vhd     [5]  PAL BRAM top (needs pal_csync_il)

└── video_bram_top.vhd      [5]  PAL+NTSC BRAM top (needs pal_csync_il)

# 3.  Option 1 — pal_tv_crt50_top  (50 Hz Progressive)

## 3.1  Compile Order

ghdl -a --std=08 tv/pal_timing.vhd

ghdl -a --std=08 tv/pal_sync_gen.vhd

ghdl -a --std=08 tv/opt1_crt50/pal_tv_crt50_top.vhd

## 3.2  Ports Quick Reference

IN:   clk, rst, sel[7:0], brightness[3:0]="1111", black_lvl[3:0]="0100"

OUT:  dac_out[3:0], hsync_o, vsync_o, active_o, blank_o

GEN:  CLK_MHZ=40, H_ACTIVE=520, V_ACTIVE_L=288, STRIPE_W=4

## 3.3  Pattern Selections & DAC Output

| sel | Pattern | Active-pixel DAC output (4-bit) |
| --- | --- | --- |
| 0x00 | Vertical Bars | 8 zones: W/S/W/S/W/S/W/BLK (65 px each) W=0xF S=alt 4/F BLK=0x4 |
| 0x01 | Horizontal Bars | Rows 0-35: WHITE(0xF), rows 36-71: Stripe, repeats x8 over 288 lines |
| 0x02 | H-Gradient | 10 steps L→R: 0x4 0x5 0x6 0x7 0x8 0x9 0xA 0xB 0xD 0xF (52 px each) |
| 0x03 | V-Gradient | 10 steps T→B: darkest top (0x4) → brightest bottom (0xF), ~29 rows/step |

## 3.4  DAC Output Visualisation

Figure: sel=0 Vertical bars — one active line pixel values

Figure: sel=1 Horizontal bars — representative active line

Figure: sel=2 Horizontal gradient — 10-step ramp

## 3.5  Waveform Capture — One Active Line

Figure: CRT-50: hsync, active, dac_out over one 64 µs H-line

## 3.6  Simulation Console Log

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:228:5:@1408500ns:(report note): === 20% contrast vertical bars PASSED (520 px) ===

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:184:5:@1408500ns:(report note): === HORIZONTAL gradient PASSED (520 px) ===

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:145:5:@1408500ns:(report note): === VERTICAL bars PASSED (520 px) ===

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:202:5:@7692500ns:(report note): === VERTICAL gradient PASSED (100 lines) ===

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:163:5:@7692500ns:(report note): === HORIZONTAL bars PASSED (100 lines) ===

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd:103:5:@20480500ns:(report note): === Simulation complete ===

simulation finished @20480500ns

EXIT:0

# 4.  Option 2 — pal_tv_interlaced_top  (PAL 625/50)

## 4.1  Compile Order

ghdl -a --std=08 tv/pal_timing.vhd

ghdl -a --std=08 tv/opt2_interlaced/pal_csync_il.vhd

ghdl -a --std=08 tv/opt2_interlaced/pal_tv_interlaced_top.vhd

NOTE: pal_csync_il.vhd must compile before pal_tv_interlaced_top.vhd.

## 4.2  Ports Quick Reference

IN:   clk, rst, sel[7:0], brightness[3:0], black_lvl[3:0]

OUT:  dac_out[3:0], hsync_o (composite sync), vsync_o (field: 0=F1 1=F2)

active_o, blank_o

GEN:  CLK_MHZ=40, H_ACTIVE=520, V_TOTAL=625, STRIPE_W=4

## 4.3  Field Structure

Two fields per frame: F1 lines 24–311 (288 active), F2 lines 336–623 (288 active). vsync_o = 0 during F1, vsync_o = 1 during F2.  csync includes 5 pre-equalising, 5 broad-sync, 5 post-equalising half-lines per field.

## 4.4  Waveforms

Figure: Interlaced: csync broad-sync + equalising region (first 10 lines, F1 field start)

Figure: Interlaced: first active line F1 — csync, active_o, field, dac_out

## 4.5  Simulation Console Log

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd:167:5:@1600500ns:(report note): === Field-1 vertical bars PASSED (520 px) ===

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd:219:5:@1600500ns:(report note): === 20% contrast Field-1 vertical bars PASSED (520 px) ===

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd:194:5:@21568500ns:(report note): === Field-2 vertical bars PASSED (520 px) ===

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd:141:5:@40187750ns:(report note): === Interlaced sync pulse shapes PASSED ===

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd:84:5:@46400500ns:(report note): === Simulation complete ===

simulation finished @46400500ns

EXIT:0

# 5.  Option 3 — pal_tv_prog25_top  (Progressive 25 Hz)

## 5.1  Compile Order

ghdl -a --std=08 tv/pal_timing.vhd

ghdl -a --std=08 tv/pal_sync_gen.vhd

ghdl -a --std=08 tv/opt3_prog25/pal_tv_prog25_top.vhd

NOTE: 25 Hz progressive – many TVs will NOT lock. Use Option 2 for real TV output.

## 5.2  Pattern Selections

Same sel=0..3 as Option 1 but 576 active lines (full PAL frame).

Figure: sel=3 Vertical gradient preview strip (top=dark → bottom=bright)

## 5.3  Simulation Console Log

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:219:5:@1664500ns:(report note): === 20% contrast vertical bars PASSED (520 px) ===

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:177:5:@1664500ns:(report note): === HORIZONTAL gradient PASSED (520 px) ===

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:140:5:@1664500ns:(report note): === VERTICAL bars PASSED (520 px) ===

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:194:5:@11148500ns:(report note): === VERTICAL gradient PASSED (150 lines) ===

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:157:5:@11148500ns:(report note): === HORIZONTAL bars PASSED (150 lines) ===

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd:100:5:@12800500ns:(report note): === Simulation complete ===

simulation finished @12800500ns

EXIT:0

# 6.  BRAM Option A — pal_tv_bram_top  (10 Patterns)

## 6.1  Compile Order

ghdl -a --std=08 tv/pal_timing.vhd

ghdl -a --std=08 tv/opt2_interlaced/pal_csync_il.vhd

ghdl -a --std=08 tv/bram/pal_tv_bram_top.vhd

# optional:

ghdl -a --std=08 tv/bram/pal_bram_loader.vhd

## 6.2  All Runtime Ports

clk, rst

sel[7:0]             pattern select  (0–9)

brightness[3:0]      white level DAC code (def "1111")

black_lvl[3:0]       black level DAC code (def "0100")

bram_wr_en           pulse 1 clk per pixel write

bram_wr_addr[18:0]   pixel address 0..BRAM_DEPTH-1

bram_wr_data         1=brightness, 0=black_lvl

bram_len[18:0]       valid pixel count (address wraps here)

ball_spd_h[3:0]      H speed px/frame 1-15  (def 3)

ball_spd_v[3:0]      V speed rows/frame 1-15 (def 2)

ball_w_i[9:0]        ball width px   (def 60)

ball_h_i[9:0]        ball height rows (def 50)

cross_h_i[9:0]       crosshatch H pitch px    (def 52)

cross_v_i[9:0]       crosshatch V pitch lines (def 58)

-- outputs --

dac_out[3:0], csync_o, line_sync_o, frame_sync_o

field_o, active_o, blank_o

## 6.3  All 10 Pattern Selections with Visual Output

| sel | Name | Active-line DAC output description |
| --- | --- | --- |
| 0x00 | Vertical Bars | 8 zones 65 px: STRIPE¦WHITE¦STRIPE¦WHITE¦STRIPE¦WHITE¦STRIPE¦BLACK |
| 0x01 | Horizontal Bars | Alternating white/stripe rows every V_ACTIVE_F/8 lines |
| 0x02 | H-Gradient | 10 grey steps 52 px each, left=dark(0x4) right=bright(0xF) |
| 0x03 | V-Gradient | 10 grey steps ~58 rows each, top=dark bottom=bright |
| 0x04 | BRAM Image | 1-bit BRAM data; addr wraps at bram_len; 0→black_lvl 1→brightness |
| 0x05 | Bouncing Ball | White rect (ball_w x ball_h) bouncing at ball_spd_h/ball_spd_v px/frame |
| 0x06 | Full White | ALL 520 px = brightness(0xF) — peak-white DAC calibration |
| 0x07 | Full Black | ALL 520 px = black_lvl(0x4) — black-level DAC calibration |
| 0x08 | Crosshatch | White pixel when (x%cross_h_i)==0 OR (line%cross_v_i)==0; else black |
| 0x09 | Centre Cross | White only at x=260 (vertical) and screen_y=288 (horizontal) |

## 6.4  Pattern Visual Strips

Figure: sel=5 Bouncing ball — line 0 frame 0 (ball at x=0)

Figure: sel=6 Full white

Figure: sel=7 Full black

Figure: sel=8 Crosshatch (cross_h=52 px)

Figure: sel=9 Centre cross (white only at px=260)

## 6.5  BRAM Address Layout (sel=4)

Stride = H_ACTIVE = 520 pixels per row.

addr      0 ..    519  → image row 0  (F1 line 0, screen row 0)

addr    520 ..   1039  → image row 1  (F2 line 0, screen row 1)

addr   1040 ..   1559  → image row 2  (F1 line 1, screen row 2)

...

addr k*1040 .. k*1040+519  → image row 2k  (F1 line k)

addr k*1040+520..k*1040+1039 → image row 2k+1 (F2 line k)

...

addr 299000..299519  → image row 575 (F2 line 287)

bram_len = 520      → one row, repeats every active line

bram_len = 1040     → row pair, repeats every row pair

bram_len = 299520   → full PAL frame

## 6.6  Waveforms

Figure: sel=5 Ball: first active line F1, dac_out shows 60 white px then black

Figure: sel=6/7: full-white then full-black active lines

Figure: sel=8 Crosshatch: first 110 px, grid lines at px=0 and px=52

Figure: sel=9 Centre cross: px 258-262, single white pixel at px=260

## 6.7  Simulation Console Logs

tb_pal_tv_bram_top:

tv/bram/tb_pal_tv_bram_top.vhd:163:5:@1600800ns:(report note): === sel=0 vertical bars PASSED (520 px) ===

tv/bram/tb_pal_tv_bram_top.vhd:191:5:@1600800ns:(report note): === BRAM F1 line 0 (image row 0, addr 0..519) PASSED ===

tv/bram/tb_pal_tv_bram_top.vhd:226:5:@21568800ns:(report note): === BRAM F2 line 0 (image row 1, addr 520..1039) PASSED ===

tv/bram/tb_pal_tv_bram_top.vhd:227:5:@21568800ns:(report note):     (px0=BLANK + px519=WHITE proves natural sequential row addressing)

tv/bram/tb_pal_tv_bram_top.vhd:137:5:@46400750ns:(report note): === Simulation complete ===

simulation finished @46400750ns

EXIT:0

tb_pal_sel5 (bouncing ball positions):

tv/bram/tb_pal_sel5.vhd:112:5:@1600500ns:(report note): === Chk1 PASS: F1 frame-0 line-0 (screen_y=0)  px0-59=WHITE  px60-519=BLACK ===

tv/bram/tb_pal_sel5.vhd:142:5:@21568500ns:(report note): === Chk2 PASS: F2 frame-0 line-0 (screen_y=1)  px0-59=WHITE  px60-519=BLACK ===

tv/bram/tb_pal_sel5.vhd:172:5:@41600500ns:(report note): === Chk3 PASS: F1 frame-1 line-0 (screen_y=0)  all BLACK (ball_y=2) ===

tv/bram/tb_pal_sel5.vhd:214:5:@41664500ns:(report note): === Chk4 PASS: F1 frame-1 line-1 (screen_y=2)  px3-62=WHITE ===

tv/bram/tb_pal_sel5.vhd:76:5:@80000500ns:(report note): === tb_pal_sel5 simulation complete ===

simulation finished @80000500ns

EXIT:0

tb_new_patterns (sel 6-9 + ball control ports):

tv/bram/tb_new_patterns.vhd:109:7:@1600550ns:(report note): === Chk1 PASS: sel=6 full white - all 520 px = WHITE ===

tv/bram/tb_new_patterns.vhd:125:7:@1664550ns:(report note): === Chk2 PASS: sel=7 full black - all 520 px = BLANK ===

tv/bram/tb_new_patterns.vhd:141:7:@1676550ns:(report note):   Chk3a PASS: px0 = WHITE (crosshatch grid line)

tv/bram/tb_new_patterns.vhd:148:7:@1679150ns:(report note):   Chk3b PASS: px26 = BLANK (crosshatch interior)

tv/bram/tb_new_patterns.vhd:155:7:@1681750ns:(report note): === Chk3 PASS: sel=8 crosshatch - px0=WHITE px26=BLANK px52=WHITE ===

tv/bram/tb_new_patterns.vhd:172:7:@1766450ns:(report note):   Chk4a PASS: px259 = BLANK (before centre)

tv/bram/tb_new_patterns.vhd:178:7:@1766550ns:(report note):   Chk4b PASS: px260 = WHITE (centre column)

tv/bram/tb_new_patterns.vhd:184:7:@1766650ns:(report note): === Chk4 PASS: sel=9 centre cross - px260=WHITE flanked by BLANK ===

tv/bram/tb_new_patterns.vhd:215:7:@3367050ns:(report note): === Chk5 PASS: ball 100-wide - px0-99=WHITE  px100-519=BLACK ===

tv/bram/tb_new_patterns.vhd:220:5:@3367050ns:(report note): === tb_new_patterns: all checks complete ===

simulation finished @3367050ns

EXIT:0

# 7.  BRAM Option B — video_bram_top  (PAL + NTSC)

## 7.1  Compile Order

ghdl -a --std=08 tv/pal_timing.vhd

ghdl -a --std=08 tv/opt2_interlaced/pal_csync_il.vhd

ghdl -a --std=08 tv/bram/video_bram_top.vhd

## 7.2  Extra Port

ntsc_mode  in std_logic   -- 0 = PAL 625/50   1 = NTSC 525/60

## 7.3  Format Comparison

| Parameter | PAL (ntsc_mode=0) | NTSC (ntsc_mode=1) |
| --- | --- | --- |
| H_TOTAL / line period | 640 (64.0 µs) | 636 (63.6 µs) |
| V_TOTAL / field rate | 625 (50 Hz) | 525 (59.94 Hz) |
| Active lines/field | 288 | 240 |
| Total active rows | 576 | 480 |
| V-gradient zone height | 58 lines/step | 48 lines/step |

## 7.4  Simulation Console Log

tv/bram/tb_video_bram_top.vhd:134:5:@1399700ns:(report note): === NTSC Chk2 PASS: F1-L0 sel=5  px0-59=WHITE  px60-519=BLACK ===

tv/bram/tb_video_bram_top.vhd:105:5:@1600500ns:(report note): === PAL Chk1 PASS: F1-L0 sel=5  px0-59=WHITE  px60-519=BLACK ===

tv/bram/tb_video_bram_top.vhd:154:5:@40000500ns:(report note): === PAL Chk3 PASS: DAC levels valid over 400000 samples ===

tv/bram/tb_video_bram_top.vhd:76:5:@80000500ns:(report note): === tb_video_bram_top complete ===

simulation finished @80000500ns

EXIT:0

# 8.  BRAM Loader — pal_bram_loader

## 8.1  Compile Order

(After pal_tv_bram_top or video_bram_top)

ghdl -a --std=08 tv/bram/pal_bram_loader.vhd

## 8.2  Ports

IN:  clk, rst, start, fifo_data, fifo_empty

OUT: done, busy, fifo_rd, bram_wr_en, bram_wr_addr[18:0], bram_wr_data

GEN: TOTAL_PX=299520, ADDR_BITS=19

## 8.3  Waveform — full 3-test sequence

Figure: BRAM loader: start → busy, normal load, FIFO stall, re-trigger, done

## 8.4  Simulation Console Log

tv/bram/tb_pal_bram_loader.vhd:146:5:@55ns:(report note): --- Test 1: Normal load ---

tv/bram/tb_pal_bram_loader.vhd:155:5:@335ns:(report note): --- Test 1 complete ---

tv/bram/tb_pal_bram_loader.vhd:161:5:@335ns:(report note): --- Test 2: Stall test ---

tv/bram/tb_pal_bram_loader.vhd:177:5:@725ns:(report note): --- Test 2 complete (stall handled) ---

tv/bram/tb_pal_bram_loader.vhd:183:5:@735ns:(report note): --- Test 3: Re-trigger ---

tv/bram/tb_pal_bram_loader.vhd:233:5:@1005ns:(report note): === Checker: all 24 writes correct ===

tv/bram/tb_pal_bram_loader.vhd:189:5:@1005ns:(report note): --- Test 3 complete ---

tv/bram/tb_pal_bram_loader.vhd:192:5:@1035ns:(report note): === All tests PASSED ===

simulation finished @1035ns

EXIT:0

# 9.  Quick-Start Configuration Guide

| Use Case | Top Entity | Files (compile order) | sel | Key Outputs |
| --- | --- | --- | --- | --- |
| CRT monitor | pal_tv_crt50_top | pal_timing pal_sync_gen pal_tv_crt50_top | 0–3 | dac_out hsync_o vsync_o |
| Real TV (interlaced) | pal_tv_interlaced_top | pal_timing pal_csync_il pal_tv_interlaced_top | 0–3 | dac_out csync_o |
| Simulation only | pal_tv_prog25_top | pal_timing pal_sync_gen pal_tv_prog25_top | 0–3 | dac_out hsync_o vsync_o |
| Custom BRAM image | pal_tv_bram_top | pal_timing pal_csync_il pal_tv_bram_top | 0–9 | dac_out csync_o field_o |
| PAL+NTSC switchable | video_bram_top | pal_timing pal_csync_il video_bram_top | 0–9 | + ntsc_mode port |

## 9.1  Step-by-Step Configuration

| Step | Action | Detail |
| --- | --- | --- |
| 1 | Compile files | Follow compile order in relevant section above. |
| 2 | Set CLK_MHZ generic | 10 / 20 / 30 / 40 — must match your board clock. |
| 3 | Wire DAC | dac_out[3:0] → 4-bit R-2R DAC (MSB = dac_out[3]). |
| 4 | Wire sync | TV output: csync_o only. CRT: hsync_o + vsync_o. |
| 5 | Set levels | brightness="1111", black_lvl="0100" (defaults are correct). |
| 6 | Reset | Hold rst=1 for ≥2 clock cycles, then rst=0. |
| 7 | Choose pattern | Write sel[7:0] = 0x00–0x09 (BRAM top) or 0x00–0x03 (other). |
| 8 | BRAM: load image | Pulse bram_wr_en per pixel; set bram_len=total pixels. |
| 9 | Ball: set parameters | Write ball_w_i, ball_h_i, ball_spd_h, ball_spd_v before rst. |
| 10 | Crosshatch: set pitch | Write cross_h_i (H spacing) and cross_v_i (V spacing) for sel=8. |

# 10.  GHDL Simulation Commands

## Run all 8 testbenches:

# Compile all sources

ghdl -a --std=08 \

tv/pal_timing.vhd tv/pal_sync_gen.vhd \

tv/opt1_crt50/pal_tv_crt50_top.vhd \

tv/opt1_crt50/tb_pal_tv_crt50_top.vhd \

tv/opt2_interlaced/pal_csync_il.vhd \

tv/opt2_interlaced/pal_tv_interlaced_top.vhd \

tv/opt2_interlaced/tb_pal_tv_interlaced_top.vhd \

tv/opt3_prog25/pal_tv_prog25_top.vhd \

tv/opt3_prog25/tb_pal_tv_prog25_top.vhd \

tv/bram/pal_bram_loader.vhd \

tv/bram/pal_tv_bram_top.vhd tv/bram/video_bram_top.vhd \

tv/bram/tb_pal_tv_bram_top.vhd tv/bram/tb_pal_sel5.vhd \

tv/bram/tb_new_patterns.vhd tv/bram/tb_video_bram_top.vhd \

tv/bram/tb_pal_bram_loader.vhd

# Elaborate & run each testbench

for tb in tb_pal_tv_crt50_top tb_pal_tv_interlaced_top \

tb_pal_tv_prog25_top tb_pal_bram_loader \

tb_pal_tv_bram_top tb_pal_sel5 \

tb_new_patterns tb_video_bram_top; do

ghdl -e --std=08 $tb

ghdl -r --std=08 $tb --assert-level=error

done

# Generate VCD waveform for a testbench:

ghdl -r --std=08 tb_pal_tv_bram_top --vcd=bram.vcd --stop-time=2ms