PAL / NTSC Video IP Core

Requirements Traceability Matrix

Rev 1.0  |  TM-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Purpose

Maps every SRS requirement to its design module (SDD) and test case (STP/STR).

# 2  Functional Requirements Traceability

| Req ID | Name | Design Module/Signal | Test Case | Result |
| --- | --- | --- | --- | --- |
| FR-001 | PAL 625/50 interlaced video | pal_csync_il.vhd, pal_tv_bram_top.vhd | TC-002, TC-004 | PASS |
| FR-002 | NTSC 525/60 runtime select | video_bram_top.vhd (ntsc_mode mux) | TC-008 | PASS |
| FR-003 | Clock flexibility 10-40 MHz | pal_timing.vhd (CLK_DIV) | TC-003, TC-008 | PASS |
| FR-004 | Pattern 0 — Vertical bars | zone_idx_v, stripe_ph_v logic | TC-001, TC-002 | PASS |
| FR-005 | Pattern 1 — Horizontal bars | zone_idx_h, stripe_ph_h logic | TC-001, TC-003 | PASS |
| FR-006 | Pattern 2 — H gradient | gzx_idx, grad_x_s process | TC-001, TC-003 | PASS |
| FR-007 | Pattern 3 — V gradient | gzy_idx, grad_y_s process | TC-001, TC-003 | PASS |
| FR-008 | Pattern 4 — BRAM image | bram_mem_0/1, bram_rd_line_base | TC-004 | PASS |
| FR-009 | BRAM write port | bram_wr_en/addr/data process | TC-004, TC-009 | PASS |
| FR-010 | Double buffering | disp_buf, swap_req, buf_swapped_s | TC-007 | PASS |
| FR-011 | Pattern 5 — Bouncing ball | ball_x/y/vx/vy, ball_on | TC-005, TC-006 | PASS |
| FR-012 | Pattern 6 — Full white | active_level mux sel=6 | TC-006 | PASS |
| FR-013 | Pattern 7 — Full black | active_level mux sel=7 | TC-006 | PASS |
| FR-014 | Pattern 8 — Crosshatch | cross_x_cnt, cross_y_cnt, cross_on | TC-006 | PASS |
| FR-015 | Pattern 9 — Centre cross | centre_on combinational | TC-006 | PASS |
| FR-016 | Brightness control | white_level_i, white_s | TC-001,TC-006 | PASS |
| FR-017 | Black level control | black_level_i, black_s | TC-006 | PASS |
| FR-018 | Sync outputs | csync_o, line_sync_o, frame_sync_o, field_o, active_o, blank_o | TC-002 | PASS |
| FR-019 | Standalone FIFO IP | tv/fifo/pal_fifo_bool.vhd — circular buf, wr/rd ptrs, count | TC-010 | PASS |

# 3  Non-Functional Requirements Traceability

| Req ID | Name | Design Evidence | Test/Review | Result |
| --- | --- | --- | --- | --- |
| NFR-001 | Pixel clock accuracy | pal_timing CLK_DIV formula | TC-001 to TC-008 (implicit) | PASS |
| NFR-002 | Sync glitch-free | pal_csync_il (combinational only) | TC-002 | PASS |
| NFR-003 | Tear-free swap | swap at V_ACT_E_F2, h_cnt=H_TOTAL-1 | TC-007 Phase 2 | PASS |
| NFR-004 | BRAM inference | ram_style=block attribute | TC-004 (synthesis check) | N/A (Vivado) |
| NFR-005 | Reset recovery | All processes reset within 3 cycles | All TCs (rst sequence) | PASS |
| NFR-006 | GHDL testbenches | All 9 tb_*.vhd files | TC-001 to TC-009 | PASS |
| NFR-007 | No division at px rate | Modulo counters, zone index | Code review | PASS |
| NFR-008 | No Xilinx primitives | Only ram_style attribute used | Code review | PASS |
| NFR-009 | Single clock domain | All processes use rising_edge(clk) | All TCs | PASS |
| NFR-010 | Modular structure | pal_timing / pal_csync_il separate | Code review | PASS |