PAL / NTSC Video IP Core

Software Design Description

Rev 1.0  |  SDD-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Architecture Overview

Figure: System Architecture Flow

The IP core uses a three-layer architecture:

Layer 1 — Timing:   pal_timing generates H/V counters at 10 MHz pixel rate.

Layer 2 — Sync:     pal_csync_il decodes H/V counters into composite sync,

field indicator, and active window signals.

Layer 3 — Patterns: pal_tv_bram_top (or video_bram_top) contains all pattern

generators, BRAM, double-buffer, and the active-level mux.

# 2  Module Decomposition

| Module | File | Purpose | Instantiates |
| --- | --- | --- | --- |
| pal_timing | tv/pal_timing.vhd | H/V counters, CLK_DIV | — |
| pal_csync_il | tv/opt2_interlaced/pal_csync_il.vhd | Interlaced composite sync (combinational) | — |
| pal_sync_gen | tv/pal_sync_gen.vhd | Non-interlaced sync (opt1/opt3) | — |
| pal_tv_crt50_top | tv/opt1_crt50/ | CRT50 progressive, sel 0–3 | pal_timing, pal_sync_gen |
| pal_tv_interlaced_top | tv/opt2_interlaced/ | PAL interlaced, sel 0–3 | pal_timing, pal_csync_il |
| pal_tv_prog25_top | tv/opt3_prog25/ | Progressive 25 Hz, sel 0–3 | pal_timing, pal_sync_gen |
| pal_tv_bram_top | tv/bram/pal_tv_bram_top.vhd | PAL + BRAM + sel 0–9 + double-buf | pal_timing, pal_csync_il |
| video_bram_top | tv/bram/video_bram_top.vhd | PAL/NTSC runtime select + BRAM + sel 0–9 | (inline counters) |

# 3  Generics

| Generic | Default | Used In | Description |
| --- | --- | --- | --- |
| H_FRONT | 16 | pal_tv_bram_top | Front porch clocks |
| H_SYNC_W | 47 | all | H-sync pulse width clocks |
| H_BACK | 57 | pal_tv_bram_top | Back porch clocks (53 for NTSC in video_bram_top) |
| H_ACTIVE | 520 | all | Active pixels per line |
| H_TOTAL | 640 | all | Total clocks per line (636 NTSC) |
| V_TOTAL | 625 | pal_tv_bram_top | Total lines per frame (525 NTSC) |
| LEVEL_SYNC | "0000" | pal_tv_bram_top | DAC code for sync tip |
| LEVEL_BLANK | "0100" | pal_tv_bram_top | DAC code for blanking floor |
| LEVEL_WHITE | "1111" | pal_tv_bram_top | DAC code for peak white |
| CLK_MHZ | 40 | all timing | Input clock frequency |
| STRIPE_W | 4 | bram tops | Stripe width for sel=0,1 |
| BRAM_DEPTH | 299520 | bram tops | BRAM address depth (520×576 PAL full frame) |

# 4  BRAM Address Map

Natural sequential row order (PAL 520×576, stride = 2×H_ACTIVE = 1040):

| Address Range | Image Row | Field | Screen Row |
| --- | --- | --- | --- |
| 0 – 519 | 0 | F1 line 0 | 0 (even) |
| 520 – 1039 | 1 | F2 line 0 | 1 (odd) |
| 1040 – 1559 | 2 | F1 line 1 | 2 (even) |
| 1560 – 2079 | 3 | F2 line 1 | 3 (odd) |
| k×1040 … k×1040+519 | 2k | F1 line k | 2k |
| k×1040+520 … k×1040+1039 | 2k+1 | F2 line k | 2k+1 |
| 299000 – 299519 | 575 | F2 line 287 | 575 (odd) |

# 5  Double-Buffer State Machine

Figure: Double-Buffer State Machine

Signals: disp_buf (front bank index), swap_req (pending swap), buf_swapped_s (output strobe)

| Condition | Action | Next State |
| --- | --- | --- |
| rst = 1 | disp_buf←0, swap_req←0 | Idle |
| buf_swap = 1 | swap_req←1 | Pending |
| v_cnt=V_ACT_E_F2, h_cnt=H_TOTAL-1, swap_req=1 | disp_buf←NOT disp_buf; swap_req←0; buf_swapped_s←1 | Idle |
| All other clocks | buf_swapped_s←0 | (unchanged) |

Write port: always writes to bram_mem_{NOT disp_buf}. Read port: always reads from bram_mem_{disp_buf}.

# 6  Active-Level Multiplexer

Figure: Active Level Multiplexer Decision Tree

Priority order (first matching condition wins):

| sel | Condition | Output |
| --- | --- | --- |
| 4 | always | bram_level (from BRAM read) |
| 5 | ball_on = 1 | white_s |
| 5 | ball_on = 0 | black_s |
| 6 | always | white_s (full white) |
| 7 | always | black_s (full black) |
| 8 | cross_on = 1 | white_s (crosshatch line) |
| 8 | cross_on = 0 | black_s |
| 9 | centre_on = 1 | white_s (centre cross) |
| 9 | centre_on = 0 | black_s |
| 2 | always | grad_x_s (H gradient level) |
| 3 | always | grad_y_s (V gradient level) |
| 0 | color_v = 1 | white_s (vertical bar) |
| 1 | color_h = 1 | white_s (horizontal bar) |
| other | always | black_s |

# 7  Ball Physics

Figure: Ball Physics Update (per frame)

Updated once per frame at v_cnt=V_ACT_E_F2, h_cnt=H_TOTAL-1:

nx = ball_x + ball_vx;  ny = ball_y + ball_vy

If nx > H_ACTIVE-ball_w: reflect (nx = 2*(H_ACTIVE-ball_w)-nx; nvx = -nvx)

If nx < 0: reflect (nx = -nx; nvx = -nvx)

Same for ny with 576-ball_h boundary (PAL)

ball_on = 1 when screen_x ∈ [ball_x, ball_x+ball_w) AND screen_y ∈ [ball_y, ball_y+ball_h)

# 8  Crosshatch Counters

cross_x_cnt: resets to 0 at H_ACT_S-1 each active line; increments each active pixel; wraps at cross_h_s.

cross_y_cnt: resets to 0 at V_ACT_S_F1 line start; increments once per active line; wraps at cross_v_s.

cross_on = 1 when cross_x_cnt=0 OR cross_y_cnt=0.