PAL / NTSC Video IP Core

Configuration Provisions Reference

Top-level entity: pal_tv_bram_top  /  video_bram_top

# 1  Generics  (Compile-time / Vivado Block Properties)

Generics are set once at synthesis time — in Vivado via the IP Customisation GUI or in the generic map of an instantiation. They cannot be changed at runtime.

| Generic | Default | Description |
| --- | --- | --- |
| H_FRONT | 16 | Horizontal front porch — clock cycles before H-sync |
| H_SYNC_W | 47 | H-sync pulse width in clock cycles |
| H_BACK | 57 | H back porch — clock cycles after H-sync, before active |
| H_ACTIVE | 520 | Active pixels per line (PAL standard) |
| H_TOTAL | 640 | Total clocks per line (H_FRONT + H_SYNC_W + H_BACK + H_ACTIVE) |
| V_TOTAL | 625 | Total lines per frame (625 = PAL, 525 = NTSC) |
| LEVEL_SYNC | "0000" | 4-bit DAC code for sync tip (0 V on R-2R ladder) |
| LEVEL_BLANK | "0100" | 4-bit DAC code for blanking floor (~0.3 V) |
| LEVEL_WHITE | "1111" | 4-bit DAC code for peak white (1 V) |
| CLK_MHZ | 40 | Input clock frequency in MHz — used for ball animation timing |
| STRIPE_W | 4 | Stripe width in pixels used by sel=0 and sel=1 patterns |
| BRAM_DEPTH | 299520 | BRAM address space (520 × 576 = 299,520). Use 524288 for power-of-2 BRAM |

ℹ  For NTSC set V_TOTAL=525, H_ACTIVE=640, H_TOTAL=858 (if using 13.5 MHz pixel clock).

# 2  Runtime Input Ports  (Changeable While Running)

## 2.1  Pattern Select

| Port | Width | Default | Description |
| --- | --- | --- | --- |
| sel | 8-bit | — | Pattern selector (see Section 3 for all values) |
| brightness | 4-bit | "1111" | White level DAC code override |
| black_lvl | 4-bit | "0100" | Black / blanking floor DAC code override |

## 2.2  BRAM Write Interface  (sel = 4)

Write pixel data directly into the internal BRAM at any time. Address is linear (row-major). The display read-pointer applies interlace correction automatically.

| Port | Width | Description |
| --- | --- | --- |
| bram_wr_en | 1-bit | Write enable — pulse high for one clock to write one pixel |
| bram_wr_addr | 19-bit | Target BRAM address (0 … bram_len−1) |
| bram_wr_data | 1-bit | Pixel value (1 = white, 0 = black) |
| bram_len | 19-bit | Total valid pixels loaded — address wraps here (default = all 1s) |

ℹ  bram_wr_en is edge-sensitive; hold address and data stable for at least one clock.

## 2.3  Bouncing Ball Control  (sel = 5)

All four ports are latched on the rising edge of rst. Assert rst for ≥ 3 clock cycles after changing any ball parameter to reload the values.

| Port | Width | Default | Description |
| --- | --- | --- | --- |
| ball_w_i | 10-bit | 60 (px) | Ball width in pixels (1 – 519) |
| ball_h_i | 10-bit | 50 (rows) | Ball height in active rows (1 – 287) |
| ball_spd_h | 4-bit | 3 (px/f) | Horizontal speed in pixels per frame (1 – 15) |
| ball_spd_v | 4-bit | 2 (row/f) | Vertical speed in rows per frame (1 – 15) |

## 2.4  Crosshatch Grid Control  (sel = 8)

Grid line spacing is set independently for horizontal and vertical. A grid line is always drawn at pixel 0 / row 0 of the active area; the spacing defines the period of the repeating grid.

| Port | Width | Default | Description |
| --- | --- | --- | --- |
| cross_h_i | 10-bit | 52 (px) | Horizontal grid spacing — pixels between vertical grid lines |
| cross_v_i | 10-bit | 58 (rows) | Vertical grid spacing — active rows between horizontal grid lines |

ℹ  Both values are clamped to a minimum of 1 internally to avoid counter wrap-around.

# 3  Pattern Selector (sel) Values

The 8-bit sel port chooses which pixel generator drives dac_out during the active picture area. Sync, blanking and back-porch levels are unaffected by sel.

| sel hex | sel dec | Pattern Name | Description |
| --- | --- | --- | --- |
| 0x00 | 0 | Vertical stripes | White stripes of width STRIPE_W on black background |
| 0x01 | 1 | Horizontal stripes | White stripes of height STRIPE_W on black background |
| 0x02 | 2 | Horizontal gradient | Luminance ramps left → right (dark to bright) |
| 0x03 | 3 | Vertical gradient | Luminance ramps top → bottom (dark to bright) |
| 0x04 | 4 | BRAM image | Displays 1-bit image written via BRAM write port (interlaced) |
| 0x05 | 5 | Bouncing ball | Solid white rectangle that bounces around screen; size/speed configurable at runtime |
| 0x06 | 6 | Full white | Every active pixel = LEVEL_WHITE (DAC output calibration) |
| 0x07 | 7 | Full black | Every active pixel = LEVEL_BLANK (DAC output calibration) |
| 0x08 | 8 | Crosshatch grid | Regular grid of white lines on black; H and V pitch set via cross_h_i / cross_v_i |
| 0x09 | 9 | Centre cross | Single white vertical line at H centre + single white horizontal line at V centre |

ℹ  sel values 0x0A and above are undefined — dac_out defaults to LEVEL_BLANK.

# 4  Output Ports

All outputs are synchronous to clk. Connect dac_out to a 4-bit R-2R resistor ladder to produce an analogue video signal. The sync outputs can drive standard video encoders or be combined externally.

| Port | Width | Description |
| --- | --- | --- |
| dac_out | 4-bit | 4-bit DAC value — connect to R-2R resistor ladder to produce analogue video |
| csync_o | 1-bit | Composite sync (H and V merged) — high during sync tip |
| line_sync_o | 1-bit | H-sync only — high during horizontal sync, never during V-sync |
| frame_sync_o | 1-bit | V-sync pulse — high during the broad-sync region of the V interval |
| field_o | 1-bit | Field indicator (0 = Field 1 / odd lines, 1 = Field 2 / even lines) |
| active_o | 1-bit | High during the active picture area (H and V both in active window) |
| blank_o | 1-bit | Composite blanking — high outside active area, but NOT during sync tip |

# 5  Quick-Reference Summary

| Category | Ports / Generics | When to Configure |
| --- | --- | --- |
| Timing | H_FRONT, H_SYNC_W, H_BACK, H_ACTIVE, H_TOTAL, V_TOTAL | Generics — set at synthesis |
| DAC levels | LEVEL_SYNC, LEVEL_BLANK, LEVEL_WHITE | Generics — set at synthesis |
| Clock | CLK_MHZ | Generic — must match actual clock |
| Pattern | sel (8-bit) | Runtime — change any time |
| Brightness | brightness, black_lvl (4-bit each) | Runtime — change any time |
| BRAM image | bram_wr_en / bram_wr_addr / bram_wr_data / bram_len | Runtime — write pixel-by-pixel |
| Ball size/speed | ball_w_i, ball_h_i (10-bit) ball_spd_h, ball_spd_v (4-bit) | Runtime — apply rst after change |
| Crosshatch pitch | cross_h_i, cross_v_i (10-bit each) | Runtime — change any time |