PAL / NTSC Video IP Core

Bug Report Log

Rev 1.0  |  BR-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Bug Report Template

| Field | Description |
| --- | --- |
| Bug ID | Unique identifier BUG-NNN |
| Date Found | Date bug was discovered |
| Severity | Critical / Major / Minor / Trivial |
| Status | Open / In Progress / Fixed / Verified / Closed |
| Module | VHDL file or tool where bug was found |
| Description | What went wrong; observed vs expected |
| Root Cause | Why it happened |
| Fix Applied | What change was made |
| Fixed In | Git commit hash |
| Verified By | Testbench or review method that confirmed fix |

# 2  Bug Log

## BUG-001 — tv/bram/tb_new_patterns.vhd

| Field | Detail |
| --- | --- |
| Bug ID | BUG-001 |
| Date Found | 2026-05-23 |
| Severity | Minor |
| Status | CLOSED |
| Module | tv/bram/tb_new_patterns.vhd |
| Description | GHDL rejected em-dash character (—, U+2014) inside VHDL report string literals. Build failed with UTF-8 parse error. |
| Root Cause | VHDL string literals must contain only 7-bit ASCII characters. Em-dash is a multi-byte UTF-8 character outside the VHDL character set. |
| Fix Applied | Replaced all em-dash occurrences with hyphen-minus (-) using sed. sed -i 's/—/-/g' tb_new_patterns.vhd |
| Commit | Inline fix during session |
| Verified By | tb_new_patterns compiles and runs without error |

## BUG-002 — tv/bram/tb_new_patterns.vhd

| Field | Detail |
| --- | --- |
| Bug ID | BUG-002 |
| Date Found | 2026-05-23 |
| Severity | Major |
| Status | CLOSED |
| Module | tv/bram/tb_new_patterns.vhd |
| Description | Testbench procedure wait_active_line() returned immediately if called while active_o was already high, landing mid-line instead of at px=0. Chk4 (centre cross) started counting from px~53 instead of px~0, causing the px260 WHITE check to read px313 (BLANK) — false FAIL. |
| Root Cause | The original procedure waited for active_o = 1 without first draining the current active window. A subsequent call mid-active period returned instantly on the current rising edge. |
| Fix Applied | Replaced procedure with next_f1_line() which: (1) drains current active window by waiting for active_o = 0, then (2) waits for next rising edge of active_o with field_o = 0. |
| Commit | Inline fix during session |
| Verified By | tb_new_patterns Chk4 PASS at correct pixel positions |

## BUG-003 — tv/bram/pal_tv_bram_top.vhd

| Field | Detail |
| --- | --- |
| Bug ID | BUG-003 |
| Date Found | 2026-05-23 |
| Severity | Minor |
| Status | CLOSED |
| Module | tv/bram/pal_tv_bram_top.vhd |
| Description | Crosshatch vertical counter (cross_y_cnt) reset condition used v_cnt = V_ACT_S_F1 - 1 which fired one line early, causing the first grid line to appear one row above the active area. |
| Root Cause | Off-by-one in reset condition: should reset at the start of the first active line, not the line before. |
| Fix Applied | Changed reset condition to v_cnt = V_ACT_S_F1 and h_cnt = 0 (fires at the very beginning of the first active line). |
| Commit | Inline fix during session |
| Verified By | tb_new_patterns Chk3a PASS: px0 of line 0 = WHITE (grid line at start) |

# 3  Open Bugs

No open bugs.

# 4  Bug Statistics

| Category | Count |
| --- | --- |
| Total bugs reported | 3 |
| Critical | 0 |
| Major | 1 |
| Minor | 2 |
| Trivial | 0 |
| Fixed & Closed | 3 |
| Currently Open | 0 |