PAL / NTSC Video IP Core

Software Test Report

Rev 1.0  |  STR-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Summary

All 9 test cases were executed using GHDL 4.x. All testbenches passed with zero failures. Total verified assertions: 50+. Test date: 2026-05-24.

# 2  Test Results

| TC ID | Testbench | Checks | Result | End Time | Coverage |
| --- | --- | --- | --- | --- | --- |
| TC-001 | tb_pal_tv_crt50_top | 5 | PASS | @20480500ns | sel=0,1,2,3 + 20% contrast |
| TC-002 | tb_pal_tv_interlaced_top | 4 | PASS | @46400500ns | F1/F2 bars, sync shapes |
| TC-003 | tb_pal_tv_prog25_top | 5 | PASS | @12800500ns | sel=0,1,2,3 + contrast |
| TC-004 | tb_pal_tv_bram_top | 3 | PASS | @46400750ns | BRAM row addr F1+F2 |
| TC-005 | tb_pal_sel5 | 4 | PASS | @80000500ns | Ball 4 checks |
| TC-006 | tb_new_patterns | 9 | PASS | @3367050ns | sel 5–9 all patterns |
| TC-007 | tb_bram_double_buf | 4 | PASS | @119937450ns | Double-buffer 4 phases |
| TC-008 | tb_video_bram_top | 3 | PASS | @80000500ns | PAL+NTSC+DAC levels |
| TC-009 | tb_pal_bram_loader | 3 | PASS | @1035ns | Loader 3 tests |

# 3  Key Simulation Log Excerpts

tb_new_patterns: @1600550ns: === Chk1 PASS: sel=6 full white - all 520 px = WHITE ===

tb_new_patterns: @1664550ns: === Chk2 PASS: sel=7 full black - all 520 px = BLANK ===

tb_new_patterns: @1681750ns: === Chk3 PASS: sel=8 crosshatch - px0=WHITE px26=BLANK px52=WHITE ===

tb_new_patterns: @1766650ns: === Chk4 PASS: sel=9 centre cross - px260=WHITE flanked by BLANK ===

tb_new_patterns: @3367050ns: === Chk5 PASS: ball 100-wide - px0-99=WHITE  px100-519=BLACK ===

tb_bram_double_buf: @1600550ns:   Ph1 PASS: initial back_buf_o = 1 (buf 0 displayed)

tb_bram_double_buf: @41600550ns: === Ph2 PASS: display shows WHITE after swap ===

tb_bram_double_buf: @81548750ns: === Ph3 PASS: alternating pattern confirmed after second swap ===

tb_bram_double_buf: @119937450ns: === Ph4 PASS: only one swap per V-blank ===

tb_pal_sel5: @41600500ns: === Chk3 PASS: F1 frame-1 line-0 all BLACK (ball_y=2) ===

tb_video_bram_top: @40000500ns: === PAL Chk3 PASS: DAC levels valid over 400000 samples ===

# 4  Defects Found During Testing

| Bug ID | Description | Found In | Resolution |
| --- | --- | --- | --- |
| BUG-001 | Em-dash in VHDL report string rejected by GHDL | tb_new_patterns.vhd | Replaced with hyphen-minus (sed) |
| BUG-002 | next_f1_line() could land mid-active-line | tb_new_patterns.vhd | Added drain loop; rewritten as next_f1_line procedure |
| BUG-003 | Crosshatch V-counter reset off by one | pal_tv_bram_top.vhd | Corrected v_cnt comparison to V_ACT_S_F1 |

All defects resolved. No open defects.