PAL / NTSC Video IP Core

Change Log & Working Features

Branch: claude/xilinx-pal-video-ip-RIDhJ     Last updated: 2026-05-24

# 1  Feature Status — All Working

| Feature | File(s) | Status | Notes |
| --- | --- | --- | --- |
| sel 0 — Vertical stripes | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | 8-zone stripe pattern; zone width = STRIPE_W generic |
| sel 1 — Horizontal stripes | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | 8-zone stripe pattern; zone height adapts PAL/NTSC |
| sel 2 — Horizontal gradient | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | 10 luminance steps left→right |
| sel 3 — Vertical gradient | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | 10 luminance steps top→bottom |
| sel 4 — BRAM image (single/double buffer) | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Natural sequential row order; interlace-corrected read; double-buffer added |
| sel 5 — Bouncing ball | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Runtime size/speed via ball_w_i, ball_h_i, ball_spd_h, ball_spd_v ports |
| sel 6 — Full white | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Every active pixel = LEVEL_WHITE; use for DAC peak-white calibration |
| sel 7 — Full black | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Every active pixel = LEVEL_BLANK; use for DAC black-level calibration |
| sel 8 — Crosshatch grid | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Runtime H/V pitch via cross_h_i / cross_v_i; modulo counter (no divider) |
| sel 9 — Centre cross | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | Single white vertical line at H_ACTIVE/2; single white horizontal at V centre |
| Ball control ports | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | ball_w_i, ball_h_i (10-bit); ball_spd_h, ball_spd_v (4-bit). Latch with rst pulse |
| Crosshatch control ports | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | cross_h_i, cross_v_i (10-bit). Change any time; min 1 enforced internally |
| BRAM double-buffering | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | buf_swap pulse → swap at next V-blank. buf_swapped strobe confirms. back_buf_o shows write bank |
| PAL/NTSC runtime select | video_bram_top.vhd | WORKING | ntsc_mode port: 0=PAL 625/50, 1=NTSC 525/60. All timing adapts at runtime |
| brightness / black_lvl ports | all top files | WORKING | 4-bit DAC level override. Clamped: black_lvl ≥ 4, black_lvl ≤ brightness |
| bram_len port | pal_tv_bram_top.vhd video_bram_top.vhd | WORKING | 19-bit. Sets read address wrap point. 520=1 row loop, 299520=full frame |
| line_sync_o / frame_sync_o outputs | all bram/interlaced files | WORKING | Separate H-sync and V-sync outputs alongside composite csync_o |
| field_o output | all bram/interlaced files | WORKING | 0 = Field 1 (odd lines), 1 = Field 2 (even lines) |
| HTML pattern generator tool | tools/bram_pattern_gen.html | WORKING | Browser-based. Patterns, text overlay, multi-zone, PNG export, 1-bit binary BRAM export |
| User & Reference Guide (Word) | tools/PAL_NTSC_Video_IP_User_Guide.docx | WORKING | Full 10-section guide with simulation logs, waveform images, DAC strip diagrams |
| Configuration Summary (Word) | tools/PAL_NTSC_Config_Summary.docx | WORKING | All generics, runtime ports, sel table, output ports, quick-reference |

# 2  Change History

| BRAM double-buffering (latest) | • Two BRAM banks: bram_mem_0 and bram_mem_1 • Write port always targets back buffer (NOT displayed) • buf_swap pulse deferred to V-blank boundary — no tearing • buf_swapped strobe confirms swap completion • back_buf_o output tells host which bank to write to • buf_swap must be a ONE-clock pulse; holding high re-arms every frame • Testbench: tb_bram_double_buf.vhd — 4 phases, all PASS | pal_tv_bram_top.vhd video_bram_top.vhd tb_bram_double_buf.vhd |
| --- | --- | --- |

| Configuration summary document | • PAL_NTSC_Config_Summary.docx: all generics, runtime ports, sel table, output ports, quick-reference summary • Generator: tools/gen_config_summary.py | tools/PAL_NTSC_Config_Summary.docx tools/gen_config_summary.py |
| --- | --- | --- |

| User Guide v2 — simulation results | • Waveform PNG images from GHDL VCD files via matplotlib • DAC output strip diagrams for all sel values • Simulation console logs (PASS/FAIL colour coded) • Generator: tools/gen_report_v2.py | tools/PAL_NTSC_Video_IP_User_Guide.docx tools/gen_report_v2.py |
| --- | --- | --- |

| sel 6–9 + ball/crosshatch ports | • sel 6: Full white (DAC peak-white calibration) • sel 7: Full black (DAC black-level calibration) • sel 8: Crosshatch grid with runtime H/V pitch ports (cross_h_i, cross_v_i) • sel 9: Centre cross (H_ACTIVE/2 vertical + V_ACTIVE/2 horizontal) • Ball ports: ball_w_i, ball_h_i (width/height), ball_spd_h, ball_spd_v (speed) • Crosshatch uses modulo counters — no hardware division • Testbench: tb_new_patterns.vhd — 5 checks, all PASS | pal_tv_bram_top.vhd video_bram_top.vhd tb_new_patterns.vhd |
| --- | --- | --- |

| HTML pattern generator tool v2 | • Text overlay: multiple entries, position, font size, colour, bold/italic • Multi-zone pattern: V/H split, per-zone type, colours, stripe width • Separate colours section with FG/BG pickers • PNG export and 1-bit binary BRAM export • Renders text before export (baked into image) | tools/bram_pattern_gen.html |
| --- | --- | --- |

| User Guide v1 — base document | • 12-section Word document • File load order, port tables, pattern descriptions • GHDL simulation commands, Vivado integration notes • Quick-start guide | tools/PAL_NTSC_Video_IP_User_Guide.docx tools/gen_report.py |
| --- | --- | --- |

| PAL/NTSC runtime select (video_bram_top) | • New top-level entity video_bram_top • ntsc_mode port: 0=PAL 625/50, 1=NTSC 525/60 • All timing signals driven combinationally from ntsc_mode • H_TOTAL=640 PAL / 636 NTSC; V_TOTAL=625 PAL / 525 NTSC | tv/bram/video_bram_top.vhd |
| --- | --- | --- |

| line_sync_o, frame_sync_o output ports | • Separate H-sync and V-sync outputs added to all variants • line_sync_o: H pulse only, suppressed during vsync • frame_sync_o: HIGH for entire vsync region • field_o: 0=F1, 1=F2 | pal_tv_bram_top.vhd video_bram_top.vhd pal_tv_interlaced_top.vhd |
| --- | --- | --- |

| sel 5 — Bouncing ball animation | • White rectangle bouncing inside active picture area • Elastic bounce (velocity reflects on wall contact) • Default: 60×50 px, speed 3h/2v px per frame | tv/bram/pal_tv_bram_top.vhd |
| --- | --- | --- |

| sel 4 — BRAM natural sequential row order | • BRAM addressed in natural image order (row 0,1,2…) • Hardware routes to correct interlaced field automatically • bram_len port for partial fill (loop shorter than full frame) • pal_bram_loader.vhd: FIFO-to-BRAM loader utility | tv/bram/pal_tv_bram_top.vhd tv/bram/pal_bram_loader.vhd |
| --- | --- | --- |

| brightness / black_lvl ports | • Replace fixed DAC levels with runtime-adjustable ports • brightness: white DAC level (4-bit, clamped ≥ 4) • black_lvl: black/blanking floor (clamped 4 ≤ x ≤ brightness) | all top-level files |
| --- | --- | --- |

# 3  Double-Buffer Quick Reference

Key rules to remember:

| Rule | Detail |
| --- | --- |
| ★ Buffer does NOT auto-swap | The display reads the same bank forever unless you explicitly pulse buf_swap. |
| ★ Pulse buf_swap for ONE clock only | Set buf_swap=1 for one clock then immediately return to 0. Holding it high re-arms after each swap — causes a new swap every frame. |
| ★ Swap executes at V-blank, not immediately | No matter when you pulse buf_swap (start/middle/end of frame), the actual bank flip happens at the last pixel of the last F2 active line. This guarantees zero tearing. |
| ★ Wait for buf_swapped before writing next frame | buf_swapped strobes 1 for one clock when swap completes. Only after this strobe is it safe to start writing the next image. |
| ★ Always check back_buf_o before writing | back_buf_o tells you which bank is currently the back (write) buffer. Always write to this bank. It automatically changes after each swap. |
| ★ Multiple pulses before V-blank = one swap | Pulsing buf_swap twice before V-blank is fine — swap_req is idempotent. Only one swap fires per V-blank period. |

# 4  Simulation Results Summary

| Testbench | File | Result | Coverage |
| --- | --- | --- | --- |
| tb_new_patterns | tb_new_patterns.vhd | 5/5 | sel=6 white, sel=7 black, sel=8 crosshatch, sel=9 centre cross, sel=5 ball ports |
| tb_bram_double_buf | tb_bram_double_buf.vhd | 4/4 | Back buf write, swap delivery, alternating pattern, one-swap-per-V-blank |
| tb_pal_tv_bram_top | tb_pal_tv_bram_top.vhd | PASS | BRAM interlaced addressing, 520×576 full frame |
| tb_pal_sel5 | tb_pal_sel5.vhd | PASS | Bouncing ball initial position and size |
| tb_pal_bram_loader | tb_pal_bram_loader.vhd | PASS | BRAM loader FIFO-to-BRAM transfer |
| tb_pal_tv_interlaced | tb_pal_tv_interlaced_top.vhd | PASS | PAL 625/50 timing, csync, field, active |
| tb_pal_tv_crt50 | tb_pal_tv_crt50_top.vhd | PASS | CRT50 progressive timing |
| tb_video_bram_top | tb_video_bram_top.vhd | PASS | PAL/NTSC runtime switch, timing constants |