PAL / NTSC Video IP Core

Interface Control Document

Rev 1.0  |  ICD-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Scope

This document defines all hardware and software interfaces of the PAL/NTSC Video IP Core, including signal levels, port definitions, timing constraints, and BRAM communication protocols.

# 2  Hardware Interfaces

## 2.1  Clock Input

| Parameter | Value | Notes |
| --- | --- | --- |
| Clock frequency | 10 / 20 / 30 / 40 MHz | Set CLK_MHZ generic to match |
| Pixel rate | 10 MHz (always) | Internal divider reduces to 10 MHz |
| Clock standard | Single-ended LVCMOS33 | Or LVCMOS18 per board voltage |
| Setup/hold | Per Xilinx FPGA datasheet |  |
| Jitter tolerance | < 500 ps RMS | Affects sync timing accuracy |

## 2.2  Reset Input

| Parameter | Value | Notes |
| --- | --- | --- |
| Polarity | Active HIGH (rst=1 asserts reset) |  |
| Type | Synchronous | Sampled on rising edge of clk |
| Minimum pulse width | 3 clock cycles | Required when changing ball parameters |
| Release | Asynchronous release allowed | Internal state initialises on next clk edge |

## 2.3  DAC Output Interface

The 4-bit dac_out port drives an R-2R resistor ladder to produce an analogue composite video signal (0–1 V into 75 Ω).

| Signal Region | dac_out[3:0] | Hex | Voltage (1 Vpp) | Description |
| --- | --- | --- | --- | --- |
| Sync tip | "0000" | 0x0 | 0.00 V | H-sync and V-sync pulse tip |
| Blanking | "0100" | 0x4 | ~0.27 V | Blanking pedestal (standard 300 mV) |
| Grey step 1 | "0101" | 0x5 | ~0.33 V | Darkest grey (gradient patterns) |
| Grey steps | "0110"–"1110" | 0x6–0xE | 0.40–0.93 V | Mid grey steps |
| White | "1111" | 0xF | 1.00 V | Peak white |

## 2.4  R-2R Ladder Recommended Values

R = 75 Ω, 2R = 150 Ω. Connect dac_out[3] (MSB) to the ladder input closest to the output. Terminate output with 75 Ω to ground.

# 3  Software / Logic Interfaces

## 3.1  Input Control Ports

| Port | Width | Dir | Default | Description |
| --- | --- | --- | --- | --- |
| clk | 1 | in | — | Pixel clock |
| rst | 1 | in | — | Synchronous reset, active HIGH |
| sel | 8 | in | — | Pattern selector (0x00–0x09) |
| brightness | 4 | in | "1111" | White level DAC code |
| black_lvl | 4 | in | "0100" | Black/blanking floor DAC code |
| ntsc_mode | 1 | in | — | video_bram_top only: 0=PAL, 1=NTSC |
| bram_wr_en | 1 | in | '0' | BRAM write enable (one pulse = one pixel) |
| bram_wr_addr | 19 | in | zeros | BRAM write address (0–BRAM_DEPTH-1) |
| bram_wr_data | 1 | in | '0' | 1-bit pixel value (0=black, 1=white) |
| bram_len | 19 | in | all-ones | Valid pixel count; address wrap boundary |
| buf_swap | 1 | in | '0' | Pulse HIGH 1 clock to request buffer swap |
| ball_spd_h | 4 | in | "0011" | Ball horizontal speed (1–15 px/frame) |
| ball_spd_v | 4 | in | "0010" | Ball vertical speed (1–15 rows/frame) |
| ball_w_i | 10 | in | "0000111100" | Ball width in pixels (default 60) |
| ball_h_i | 10 | in | "0000110010" | Ball height in rows (default 50) |
| cross_h_i | 10 | in | "0000110100" | Crosshatch H spacing px (default 52) |
| cross_v_i | 10 | in | "0000111010" | Crosshatch V spacing rows (default 58) |

## 3.2  Output Ports

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| dac_out | 4 | out | 4-bit DAC value — connect to R-2R ladder |
| csync_o | 1 | out | Composite sync (H+V merged, HIGH = sync tip) |
| line_sync_o | 1 | out | H-sync only — suppressed during V-sync region |
| frame_sync_o | 1 | out | V-sync pulse — HIGH for entire broad-sync region |
| field_o | 1 | out | Field: 0=F1 (odd lines), 1=F2 (even lines) |
| active_o | 1 | out | HIGH during active picture area |
| blank_o | 1 | out | Composite blanking (HIGH outside active, not during sync) |
| buf_swapped | 1 | out | Strobes HIGH 1 clock when double-buffer swap completes |
| back_buf_o | 1 | out | Index of current back (write) BRAM bank (0 or 1) |

# 4  BRAM Write Protocol

The BRAM write port operates independently of the pixel clock. The host may write at any time.

Figure: BRAM Write Protocol

| Step | Action | Notes |
| --- | --- | --- |
| 1 | Set bram_wr_addr to target pixel address (0–BRAM_DEPTH-1) | Row-major order |
| 2 | Set bram_wr_data to 0 (black) or 1 (white) |  |
| 3 | Assert bram_wr_en = 1 for ONE clock cycle | One pixel written per pulse |
| 4 | Deassert bram_wr_en = 0 | Next address may be set immediately |
| 5 | Repeat for each pixel | Sequential writes are independent |

# 5  Double-Buffer Handshake Protocol

Figure: Double-Buffer Handshake Protocol

| Step | Signal | Action |
| --- | --- | --- |
| 1 | back_buf_o | Read back_buf_o — this is the bank to write to |
| 2 | bram_wr_* | Write complete new frame into back buffer |
| 3 | buf_swap | Pulse buf_swap = 1 for exactly ONE clock, then return to 0 |
| 4 | buf_swapped | Wait for buf_swapped = 1 (one clock strobe at V-blank) |
| 5 | back_buf_o | Read back_buf_o again — now points to the new back buffer |
| 6 | — | Repeat from step 2 for next frame |

WARNING: Holding buf_swap HIGH causes a new swap request every clock. Always return buf_swap to 0 within one clock cycle.

# 6  FIFO Interface  (pal_fifo_bool)

Standalone 1-bit (Boolean) synchronous FIFO IP. Depth 600,000 entries. Not connected to the BRAM display path. Single clock domain, synchronous reset.

## 6.1  Input Ports

| Port | Width | Dir | Default | Description |
| --- | --- | --- | --- | --- |
| clk | 1 | in | — | System clock (any rate; shared with other logic) |
| rst | 1 | in | — | Synchronous active-HIGH reset |
| wr_en | 1 | in | '0' | Write enable — pulse HIGH 1 clock to write |
| wr_data | 1 | in | '0' | 1-bit Boolean value to write |
| rd_en | 1 | in | '0' | Read enable — pulse HIGH 1 clock to advance pointer |

## 6.2  Output Ports

| Port | Width | Dir | Description |
| --- | --- | --- | --- |
| full | 1 | out | HIGH when FIFO is full; wr_en ignored |
| wr_ack | 1 | out | Strobes HIGH 1 clock when write accepted |
| rd_data | 1 | out | Registered read output — valid 1 clock after rd_en |
| empty | 1 | out | HIGH when FIFO is empty; rd_en ignored |
| rd_valid | 1 | out | Strobes HIGH 1 clock when rd_data updated |
| level | 20 | out | Fill count 0..DEPTH (unsigned) |
| almost_full | 1 | out | HIGH when level >= DEPTH - AF_THRESH (default 4) |
| almost_empty | 1 | out | HIGH when level <= AE_THRESH (default 4) |

## 6.3  Generics

| Generic | Default | Description |
| --- | --- | --- |
| DEPTH | 600000 | Number of 1-bit entries; needs 19 BRAM36 tiles |
| AF_THRESH | 4 | Almost-full distance from top |
| AE_THRESH | 4 | Almost-empty distance from zero |

## 6.4  BRAM Resource Usage

| Parameter | Value |
| --- | --- |
| Entry width | 1 bit (std_logic) |
| Depth | 600,000 entries |
| Total storage | 600,000 bits |
| BRAM36 tiles needed | 19 (600,000 ÷ 32,768 = 18.3 → 19) |
| Available (cRIO-9056) | 105 BRAM36 tiles |
| FIFO usage | 18% — well within budget |
| Read latency | 1 clock cycle (registered output) |