PAL / NTSC Video IP Core

Software Test Plan

Rev 1.0  |  STP-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Purpose

This plan describes the test strategy, test environment, test cases, and pass/fail criteria for all modules of the PAL/NTSC Video IP Core.

# 2  Test Environment

| Item | Specification |
| --- | --- |
| Simulator | GHDL 4.x, VHDL-2008 standard (--std=08) |
| Compile cmd | ghdl -a --std=08 <dependencies> <dut> <tb> |
| Elaborate cmd | ghdl -e --std=08 <entity> |
| Run cmd | ghdl -r --std=08 <entity> --assert-level=error [--vcd=<file>] |
| Pass criteria | Process exits 0 with no severity failure or error reports |
| Fail criteria | Any "severity failure" assertion terminates with non-zero exit |
| Host OS | Linux (Ubuntu 22.04 or later) |
| Python | 3.10+ for log analysis and report generation |

# 3  Test Cases

| TC ID | Testbench | Requirements | Description |
| --- | --- | --- | --- |
| TC-001 | tb_pal_tv_crt50_top | FR-004,FR-005,FR-006,FR-007,FR-016 | Verify all 4 base patterns (sel 0–3) in CRT50 progressive mode with contrast control |
| TC-002 | tb_pal_tv_interlaced_top | FR-001,FR-004,FR-005,FR-018,NFR-002 | Verify interlaced PAL sync waveform, F1/F2 active windows, and base patterns |
| TC-003 | tb_pal_tv_prog25_top | FR-003,FR-004,FR-005,FR-006,FR-007 | Verify progressive 25 Hz mode with all base patterns and brightness control |
| TC-004 | tb_pal_tv_bram_top | FR-008,FR-009,NFR-004 | Verify BRAM natural sequential row addressing across F1 and F2 lines |
| TC-005 | tb_pal_sel5 | FR-011,FR-016 | Verify bouncing ball initial position, frame-1 displacement, and boundary |
| TC-006 | tb_new_patterns | FR-012,FR-013,FR-014,FR-015,FR-011,FR-016,FR-017 | Verify sel 6 (white), 7 (black), 8 (crosshatch px0/26/52), 9 (centre cross px259/260/261), ball w=100 |
| TC-007 | tb_bram_double_buf | FR-010,NFR-003 | Verify: back buf unchanged before swap; swap delivers new image; alternating pattern; single swap per V-blank |
| TC-008 | tb_video_bram_top | FR-002,FR-003,FR-011,NFR-001 | Verify PAL/NTSC runtime switch, ball in both modes, DAC levels valid over 400k samples |
| TC-009 | tb_pal_bram_loader | FR-009 | Verify BRAM loader: normal write, stall recovery, re-trigger without reset |

# 4  Test Execution Flow

Figure: Test Execution Process

# 5  Pass / Fail Criteria

PASS: ghdl exits 0 AND all "PASS" report messages appear in log.

FAIL: Any "FAIL" report message OR ghdl exit code ≠ 0 OR severity failure.

REGRESSION: Re-run all TCs after every RTL change. All must pass before commit.