PAL / NTSC Video IP Core

Software Requirements Specification

Rev 1.0  |  SRS-001  |  2026-05-24

Branch: claude/xilinx-pal-video-ip-RIDhJ

# 1  Introduction

This document specifies all functional and non-functional requirements for the PAL/NTSC Video IP Core. Requirements are identified by unique IDs and shall be traced to design (SDD) and test (STP/STR) documents.

# 2  System Overview

The Video IP Core is a synthesisable VHDL IP block that generates standard-definition PAL (625/50) or NTSC (525/60) interlaced composite video from an internal 1-bit-per-pixel BRAM image store. The IP targets Xilinx Artix-7 / Kintex-7 FPGAs and drives a 4-bit R-2R DAC.

# 3  Functional Requirements

| ID | Name | Requirement |
| --- | --- | --- |
| FR-001 | Video Standard | The IP shall generate PAL 625-line 50 Hz interlaced video with composite sync conforming to CCIR-B standard. |
| FR-002 | NTSC Mode | video_bram_top shall support NTSC 525-line 59.94 Hz mode selectable at runtime via ntsc_mode port without re-synthesis. |
| FR-003 | Clock Flexibility | The IP shall accept input clocks of 10, 20, 30, or 40 MHz and produce a 10 MHz effective pixel rate via an internal divider. |
| FR-004 | Pattern 0 — Vertical Bars | sel=0 shall display 8 vertical zones of alternating STRIPE/WHITE/BLACK patterns, each 65 pixels wide. |
| FR-005 | Pattern 1 — Horizontal Bars | sel=1 shall display 8 horizontal zones matching the ZONE_TABLE, each 36 active rows high (PAL). |
| FR-006 | Pattern 2 — H Gradient | sel=2 shall display a 10-step horizontal luminance gradient from black_lvl to brightness across 520 pixels. |
| FR-007 | Pattern 3 — V Gradient | sel=3 shall display a 10-step vertical luminance gradient from black_lvl to brightness across 288 active rows per field. |
| FR-008 | Pattern 4 — BRAM Image | sel=4 shall display a 1-bit-per-pixel image stored in internal BRAM, addressed in natural sequential row order with automatic interlace correction. |
| FR-009 | BRAM Write Port | The IP shall provide a synchronous 19-bit-addressed 1-bit-data write port operable at any time independent of the display scan. |
| FR-010 | Double Buffering | The IP shall support tear-free BRAM image updates via two BRAM banks. A buf_swap pulse shall cause the banks to exchange at the next V-blank boundary only. |
| FR-011 | Pattern 5 — Bouncing Ball | sel=5 shall display a white rectangle bouncing inside the active area with elastic boundary reflection. Size and speed shall be settable at runtime via ports. |
| FR-012 | Pattern 6 — Full White | sel=6 shall output LEVEL_WHITE on every active pixel for DAC peak-white calibration. |
| FR-013 | Pattern 7 — Full Black | sel=7 shall output LEVEL_BLANK on every active pixel for DAC black-level calibration. |
| FR-014 | Pattern 8 — Crosshatch | sel=8 shall display a crosshatch grid with runtime-configurable horizontal and vertical line spacing. |
| FR-015 | Pattern 9 — Centre Cross | sel=9 shall display a single white vertical line at horizontal centre and a single white horizontal line at vertical centre. |
| FR-016 | Brightness Control | A 4-bit brightness port shall override the white DAC level for all patterns. Value shall be clamped to [4, 15]. |
| FR-017 | Black Level Control | A 4-bit black_lvl port shall set the active black/blanking floor for all patterns. Value clamped to [4, brightness]. |
| FR-018 | Sync Outputs | The IP shall provide composite sync (csync_o), H-sync-only (line_sync_o), V-sync (frame_sync_o), field indicator (field_o), active (active_o), and blanking (blank_o) outputs. |
| FR-019 | Standalone FIFO IP | A standalone 1-bit (Boolean) synchronous FIFO IP (pal_fifo_bool) shall be provided with depth 600,000 entries, full/empty/almost_full/almost_empty flags, wr_ack, rd_valid strobes, and a 20-bit fill-level output. It shall be independent of the BRAM display path. |

# 4  Non-Functional Requirements

| ID | Name | Requirement |
| --- | --- | --- |
| NFR-001 | Pixel clock accuracy | H_TOTAL × line_rate shall be within ±0.1% of PAL/NTSC standard. |
| NFR-002 | Sync glitch-free | pal_csync_il shall be purely combinational; no glitches on csync output. |
| NFR-003 | Tear-free swap | Double-buffer swap shall complete entirely within the V-blank interval. |
| NFR-004 | BRAM inference | BRAM arrays shall carry Xilinx ram_style=block attribute for automatic BRAM36 inference. |
| NFR-005 | Reset recovery | All state machines shall reach a defined initial state within 5 clock cycles of rst deassertion. |
| NFR-006 | Simulation | All modules shall have GHDL-compatible VHDL-2008 self-checking testbenches. |
| NFR-007 | No division at pixel rate | Pattern generators shall use only addition and comparison; no division or multiplication in pixel-rate paths. |
| NFR-008 | Portability | RTL shall contain no Xilinx-specific primitives other than the ram_style attribute. Synthesis tool handles BRAM mapping. |
| NFR-009 | Single clock domain | All design logic shall operate in the single clk domain. No asynchronous paths except BRAM read. |
| NFR-010 | Modular structure | Timing and sync generation shall be separate instantiated modules to allow reuse. |

# 5  Constraints

| Constraint | Value |
| --- | --- |
| Target FPGA family | Xilinx 7-series (Artix-7, Kintex-7) or later |
| VHDL standard | VHDL-2008 (--std=08) |
| Simulation tool | GHDL 4.x or later |
| Synthesis tool | Vivado 2020.x or later |
| DAC resolution | 4-bit (R-2R resistor ladder) |
| Base pixel clock | 10 MHz (100 ns per pixel) |
| Active resolution | 520 × 576 px (PAL), 520 × 480 px (NTSC) |
| BRAM depth (PAL) | 299,520 bits (520 × 576), ~10 × BRAM36 |